SECTION I 	PRODUCT IDENTIFICATION AND USE

Supplier 	: NATIONAL CONCRETE ACCESSORIES	- H M I S -

Address     	        : 14305-128 AVE.                          			        |Health                                   : 1

: Edmonton, AB   T5L 3H3	|Fire 	: 3

|Reactivity 	: 0

Telephone # 	        : (780) 451-1212             			        |Person Protection     	        :      |

Emergency # 	:

(Hazard Rating: 0=Least,1=Slight,2=Moderate,3=High,4=Extreme,*=Chronic)

Product Class 	: Class B2, Flammable Liquids, Class D2B, Toxic Materials

Product Identification Number: N/A

Product Identifier	: AR PREMIUM ACRYLIC SEALER

Product Use	: Concrete Sealer

SECTION II 	HAZARDOUS INGREDIENTS

% By 	ACGIH

No. Hazardous Ingredient(s)* 	CAS# 	Weight 	    LD50        LC50          TLV/TWA        TLV/Ceiling 	TLV/STEL 	SKIN

1. Proprietary Solvent Blend

* A more complete disclosure will be provided to a physician or nurse in the event of a medical emergency. None of the components of this product are recognized as carcinogenic.

Component data is defined in accordance with Sub-paragraph 13 (a) (i) to (iv) of the Hazardous Product Act.

SECTION III 	PHYSICAL DATA

Physical State 	: Liquid	Vapour Density 	: >1 (Air=1)

Specific Gravity 	: 0.86 @ 15 degrees C	Evaporation Rate	: >1 (TBAC=1)

Odour/Appearance	: Colorless Liquid, Solvent Odor	Percent Volatile	: 82

Vapour Pressure	: Not Established	Freezing Point 	: Not Established

pH 	: Not Applicable	Boiling Point 	: Not Established

Odour Threshold 	: Not Established                                               VOC Content	: 334 g/L

- Coefficient of water/oil distribution: Not Established

SECTION IV 	FIRE AND EXPLOSION DATA

Conditions of Flammability:  Ignition Sources                                                               Autoignition temperature: Not Established

Means of extinction: Foam, CO2, Dry Chemical		Sensitivity to mechanical impact: No

Flash point and method: -20 Degrees C (Estimate)	Sensitivity to static discharge: Yes

Flammability limits: 	LEL: N/E	UEL: N/E

Hazardous combustion products: Carbon Monoxide, Carbon Dioxide, Various Hydrocarbons, and incomplete combustion products.

- SECTION V 	REACTIVITY DATA

- Chemical stability: Stable

- If no, under which conditions:

- Incompatibility (materials to avoid): Strong Oxidizing Agents

- Conditions of reactivity: None

Hazardous decomposition products: Carbon Monoxide, Carbon Dioxide, Various Hydrocarbons, and incomplete combustion products.

SECTION VI 	TOXICOLOGICAL PROPERTIES

- Route(s) of entry: Eye contact, skin contact, and inhalation.

- Effect(s) of acute exposure to product: Skin, eye, and respiratory irritation. Central nervous depression may also occur.

- EYE CONTACT: Direct contact may cause mild to moderate irritation. Symptoms include stinging, tearing, and redness.

SKIN CONTACT: May cause mild to moderate skin irritation. Prolonged/repeated contact may cause drying of the skin.

INHALATION: Exposure to vapors may cause mild to moderate irritation of the respiratory tract. Symptoms of transient central nervous system depression may occur.

INGESTION: May cause mild to moderate irritation of the digestive tract.Transient central nervous system depression may also occur.

SIGNS AND SYMPTOMS: Symptoms of eye irritation include pain, tearing, redness, and swelling. Symptoms of skin irritation include reddening, swelling, and rash. Symptoms of respiratory irritation include runny nose, sore throat, coughing, chest discomfort, shortness of breath, and reduced lung function. Symptoms of gastrointestinal irritation include sore throat, abdominal pain, nausea, vomiting, and diarrhea. Transient central nervous system depression may be evidenced by headache, dizziness, nausea, and symptoms of intoxication; in extreme cases, unconsciousness and death may occur.

AGGRAVATED MEDICAL CONDITIONS: None Known.

OTHER HEALTH EFFECTS: Chronic exposures or high levels of exposure may cause liver, kidney, and adrenal gland changes.

- Exposure limits:

- Effect(s) of chronic exposure to product:

Carcinogenicity: No 	Reproductive toxicity: No

Teratogenicity: No	Mutagenicity: No	Synergistic Properties: No

SECTION VII 	PREVENTIVE MEASURES

Personal protective equipment: Safety glasses/goggles, chemical-resistant gloves and clothing as needed.

- Engineering controls: Ventilation to maintain exposure levels below exposure limits.

- Leak and spill procedures: Large Spills>>Evacuate hazard area of unprotected personnel. Utilize appropriate personal protective equipment. Remove source of spill if safe to do so. Remove/extinguish ignition sources. Dike and contain. If vapor cloud forms, water fog may be used to suppress. Contain run-off. Remove with vacuum trucks or pump to storage/salvage container for proper disposal. Small Spills>> Utilize an appropriate absorbent material and place in marked containers for disposal.

- Waste disposal: Observe all Provincial, Federal, State, and local regulations regarding proper disposal.

- Handling procedures/equipment: Handle as a flammable product. Keep containers closed when not in use.

Storage requirements: Store in a cool, dry area away from ignition sources.

Special shipping information: Un1866, Resin Solution, 3, II

SECTION VIII 	FIRST AID MEASURES

- EYE CONTACT: Immediately flush eyes with water for fifteen (15) minutes. If symptoms persist, seek medical attention.

- SKIN CONTACT: Remove contaminated shoes and clothing. Cleanse affected areas with mild soap and water. If symptoms persist, seek medical attention.

INHALATION: If respiratory symptoms develop, move victim from exposure source and into fresh air. Treat symptomatically. If symptoms persist, seek medical attention.

INGESTION: Seek medical attention. If individual is drowsy or unconscious, do not give anything by mouth. Place individual on left side with head down. Consult a physician, medical facility, or poison control center for advice on whether to induce vomiting.